Python Converted Code

import numpy as np

import boto3  # AWS SDK for Python

import json

from scipy import signalk

from pywt import cwt

from scipy.signal import stft

import pickle

import time

# AWS IoT Core Configuration

AWS_IOT_ENDPOINT = "your-aws-iot-endpoint.amazonaws.com"

AWS_IOT_TOPIC_SUBSCRIBE = "emg/data"

AWS_IOT_TOPIC_PUBLISH = "emg/commands"

# Initialize AWS IoT MQTT Client

client = boto3.client('iot-data', region_name='your-region')  # Replace 'your-region'

def vmd(emg_signal, num_imfs=5):

"""Perform Variational Mode Decomposition"""

from vmdpy import VMD

alpha = 2000       # Moderate bandwidth constraint

tau = 0.           # Noise-tolerance (no strict fidelity enforcement)

DC = 0             # No DC part imposed

init = 1           # Initialize omegas uniformly

tol = 1e-7

imf, _, _ = VMD(emg_signal, alpha, tau, num_imfs, DC, init, tol)

return imf

def extract_emg_features(filtered_emg, fs):

"""Extract time and frequency domain features from EMG signal"""

features = []

# Time Domain Features

mean_value = np.mean(filtered_emg)

std_value = np.std(filtered_emg)

rms_value = np.sqrt(np.mean(filtered_emg**2))

zero_crossings = np.sum(np.diff(np.sign(filtered_emg)) != 0)

features.extend([mean_value, std_value, rms_value, zero_crossings])

# Wavelet Transform Features using CWT

scales = np.arange(1, len(filtered_emg)+1)

wt, _ = cwt(filtered_emg, scales, 'morl')

energy_wavelet = np.sum(np.abs(wt)**2, axis=1)

features.extend(energy_wavelet)

# Frequency Domain Features using Short-Time Fourier Transform (STFT)

window_size = int(fs * 0.1)

overlap_size = int(window_size * 0.5)

fft_length = max(1024, window_size)

f, t, S = stft(filtered_emg, fs=fs, window='hamming', nperseg=window_size,

noverlap=overlap_size, nfft=fft_length)

power_spectral_density = np.abs(S)**2

mean_psd = np.mean(power_spectral_density, axis=1)

features.extend(mean_psd)

return features

def classify_emg(features):

"""Load trained ANN model and predict action"""

with open('trained_ann_model.pkl', 'rb') as f:

ann_model = pickle.load(f)

predicted_label = ann_model.predict([features])

return round(predicted_label[0])

def process_emg_data(data):

"""Process incoming EMG data and classify"""

fs = 10000  # Sampling frequency (adjust as necessary)

# Apply bandpass filter

b, a = signal.butter(4, [70 / (fs / 2), 2500 / (fs / 2)], btype='bandpass')

filtered_data = signal.lfilter(b, a, data)

# Perform VMD

imfs = vmd(filtered_data)

# Extract features from the first IMF (adjust as needed)

features = extract_emg_features(imfs[0], fs)

# Classify EMG signal

predicted_label = classify_emg(features)

return predicted_label

def send_command_to_esp32(command):

"""Send command to ESP32 via AWS IoT"""

payload = {"command": command}

client.publish(topic=AWS_IOT_TOPIC_PUBLISH,

qos=1,

payload=json.dumps(payload))

def main():

print("Listening for EMG data...")

while True:

try:

# Simulate receiving EMG data from AWS IoT Core (replace with actual subscription logic)

response = client.get_thing_shadow(thingName="your-esp32-thing-name")

payload_str = response['payload'].read().decode('utf-8')

payload_json = json.loads(payload_str)

if "state" in payload_json and "reported" in payload_json["state"]:

emg_data_raw = payload_json["state"]["reported"].get("emg_data", [])

if not emg_data_raw:

continue

emg_data_raw = np.array(emg_data_raw)  # Convert to NumPy array

# Process EMG data and classify action

predicted_label = process_emg_data(emg_data_raw)

# Determine command based on classification result

if predicted_label == 2:  # Grip action

send_command_to_esp32("G")

elif predicted_label == 1:  # Tripod action

send_command_to_esp32("T")

else:

send_command_to_esp32("S")

time.sleep(0.05)  # Small delay for stability

except Exception as e:

print(f"Error: {e}")

time.sleep(1)  # Retry after delay

if __name__ == "__main__":

main()